+-----------------------+-----------------------------------------------------------------+-----------------------+

|         Code          |                           Description                           |     Pinpoint Test     |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|     No Indicator      | Inoperative air bag indicator circuit                           |           A           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

| Continuous Indicator  | Diagnostic monitor disconnected or inoperative                  |           B           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          12           | Loss of battery power to air bag firing circuit                 |           C           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          13           | Front crash sensor or air bag circuit shorted to ground         |           D           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          21           | LH B-Pillar safing sensor not mounted to vehicle properly       |           E           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          22           | Safing sensor shorted or stuck closed                           |           F           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          23           | Open connection in safing sensor feed circuit                   |           G           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          24           | Safing sensor diagnostic circuit open                           |           H           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          31           | All system air bag(s) open                                      |           I           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          32           | High resistance or open circuit in driver air bag circuit       |           J           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          33           | High resistance or open circuit in passenger air bag circuit    |           K           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          34           | Low resistance in driver air bag circuit                        |           L           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          41           | Open connection in forward crash sensor circuit                 |           M           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          44           | RH front crash sensor not mounted to vehicle properly           |           N           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          45           | Center front sensor not mounted to vehicle properly             |           O           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          46           | LH front crash sensor not mounted to vehicle properly           |           P           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          51           | Diagnostic monitor internal thermal fuse open                   |           Q           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          52           | Backup power supply fault                                       |           R           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|          53           | Diagnostic monitor internal fault                               |           S           |

|-----------------------+-----------------------------------------------------------------+-----------------------|

|         99(1)         | All crash sensors disconnected                                  |           T           |

|-----------------------+-----------------------------------------------------------------+-----------------------+

(1) - Air bag indicator flashing very fast continuously.